	
	[image: image1.jpg]





Position lamp
Socket is small and add extra Red color wire to FCS-12 Relay position module pictures

	FCS-12 Relay

	a





	b

[image: image3.jpg]



	c.

[image: image4.jpg]



	d.






	[image: image6.jpg]



	



	[image: image8.jpg]



	

	a. Original socket
	b. change to 2 filament 

socket
	c. turn on night light bulb 

will on
	d. FCS12 relay

	
	Rough size of FCS12
	
	

	Corner Light内のdouble filament will be best.
socket have 4 cables

1. Red wire – to headlamp mark lamp

2. Black – To headlamp ground
3. Green wire – To turn light
4. Yellow wire – to park (output ) 
Installation

a. Change the original socket 

1156 or 3156 socket to 

dual filament 1157 or 3157 so you will 2 hot wire.

Connect the Red wire to the headlamp marking lamp Positive.

Connect the black wire to headlamp marking lamp negative. 

Connect the Green wire to turn light.

Connect the yellow wire to park.

定位燈控制器
將原單芯方向燈改成雙芯燈泡
在開啟小燈時.5CP燈絲亮起
打方向燈時.5CP燈絲熄滅.32CP閃爍亮起.間歇時小燈不亮
方向燈復位時.約1秒時差後5CP小燈亮起
 功能是提供某些車款小燈位置不明顯.開啟時不易注意到而設計
且控制方向燈閃爍間歇時.不會因為小燈半亮而影響方向燈辨識度
 如有問題 請與我司連絡 奧堤可 陳先生
 




